# LAM	ENGINEERING 12	SPRING 2005

PHYSICAL SYSTEMS ANALYSIS

LABORATORY 1:  IMPULSE RESPONSE

## Purpose

In this lab, you will verify the impulse response of a first and second order circuit, and compare theoretical predictions to experimental results for a simulated impulsive voltage input.  You will also document the pulse response of the circuits you characterize, because an ideal impulse (infinite amplitude and infinitesimal duration) is impossible to generate in the laboratory.

# Procedure

- First order circuit

Design and characterize a first order RC circuit driven by the pulse generator (that supplies a voltage) and measure the output voltage when the input voltage is a “short” pulse.  Measure the output across the resistor.

To determine the values of R and C, think about what we mean by a “short” pulse – i.e. a pulse whose duration is short compared to what?  In order to guide you in your selection, you will probably want to check the pulse generator to see what range of pulse durations it is capable of producing.  I suggest that you also observe these on the oscilloscope before choosing values for the duration of the input pulse, R, and C.  Just because the instrument claims to be capable of producing such an output, it may not be ideal for all possible ranges of values.

Obtain printouts that show both the input and output voltage on the same screen.  Show the results on two different time scales:  one that shows the pulse to be very short – i.e. a laboratory version of the impulse, and another that shows the actual pulse with measurable length, so that you can see the response of the circuit over the duration of the pulse.

- Second order circuit

- Again using the pulse generator as a source, connect a series RLC circuit with the capacitor connected so that you can measure the output voltage across it.  Choose values of R, L, and C so that the circuit is overdamped.  Obtain printouts of the input and output voltage, again on the two different time scales described above.

- Repeat part II.1 with the output voltage measured across the inductor.

# LAM	ENGINEERING 12	SPRING 2005

# PHYSICAL SYSTEMS ANALYSIS	page 2

LABORATORY 1:  IMPULSE RESPONSE (CONTINUED)

- Electronic Work Bench, EWB

Use EWB to simulate the responses in parts I, II.1, and II.2.

Report

1.	Please follow the format for a formal report for this laboratory experiment.

2.	Include derivations of the impulse response for each of the output voltages measured.  Use the input pulse duration and height to predict the amplitude (V0) of the input impulse voltage (vin(t) = V0(t)) in the circuits you have examined.

3.	Discuss the pulse response of each of the output voltages measured.  Qualitatively describe the pulse response of each output voltage.

4.	Derive the pulse response.  Hint:  you might want to think of the input voltage as a combination of two step inputs separated in time by t, where t is the duration of the pulse.

5.	Compare theory to experiment.  For example, compare predicted and measured time constants, and include percentage errors.  When there are discrepancies, please elaborate on the possible reasons for these differences, and how these effects are expected to affect the results.